Liquid S0 H2-Target-Parameters

Total amount of Hydrogen in the system:	 97 liters at NTP (Deuterium: 103 liters at NTP)

H2-purity:		6.0

H2-liquid Volumes:

Target:			7 cm3

= 0.49g H2 liqu.  5.8 l H2 at NTP

L = 10 mm

(H2-mol= 2.0158, Dens. = 0.07 g/cm3

MolVol 273.15K 1bar: 22.414 103 cm3 mol-1)

Conn. tubes:		9 cm3

Bottom of condenser:	29 cm3

Distances:

TS2QT11:		1692	mm

TA 1:			-580	mm

TA 2:			509	mm

Windows:		Ti, 30 mm , 20 m = 9 mg/cm2

Crash Pressure:	7.5 bar

sealed with Metal Helicoflex Al-coated

Calibration-Target:	78m Ti = 35 mg/cm2

+ 10 x Mylar/Al ( 6 m incl. 10 g/cm2 Al)

Distance to Center H2-Target: +82 mm

Stripping-Foil:	70m Nb ( = 60 mg/cm2 )

Distance to Center H2-Target: +142 mm

Isol.-Vacuum:		Turbomolekular-Pump Leybold Ser. 50  33l/s

Roughing Pump  Alcatel 15m3/h

Questions?      	 K.Burkard Tel. 2732 / k.burkard@gsi.de

h2tapar2.doc